- SCEW-2* – 10 Gallon self contained portable eye wash and/or drench hose unit with stainless steel pressurized tank. Unit shall have (2) polypropylene ‘GS Plus’ spray heads with integral “flip-top” dust covers and filters.  ½” IPS chrome plated ball brass ball valve with powder coated cast aluminum flag handle.  Delivers .5 GPM for 15 Minutes.  Unit shall include ANSI compliant sign.

- Drench Hose: G5011 with 6ft. PVC hose

- Performance:  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2004, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1512