- EF-27* – Wall mounted eye/face wash less bowl with ½” U.S. made chrome plated brass stay open ball valve, powder coated cast aluminum flag handle and wall bracket. Unit shall have (2) polypropylene ‘FS Plus’ spray heads with integral “flip-top” dust covers, filters and 3.2-GPM flow control orifices mounted on a chrome plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2004, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1771