- EF-39* – Pedestal mounted corrosion resistant eye/face wash with ABS plastic bowl, ½” U.S. made stay open PVC coated brass ball valve, powder coated cast aluminum flag handle and PVC wall bracket.  Unit shall have (4) polypropylene ‘GS Plus’ spray heads with integral “flip-top” dust covers, filters and 1.8-GPM flow control orifices mounted on a PVC coated brass head assembly. Unit shall include ANSI compliant sign.

- Performance:  Unit shall be hydrostatically tested to meet or exceed ANSI Z358.1 – 2004, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1796