- SS-27* - Combination eye/face wash and shower safety station with hand and foot control, ABS plastic shower head (optional stainless steel shower head ”SSH”), stainless steel bowl, powder coated cast aluminum flag handle and floor flange, 1 ¼” IPS Schedule 40 galvanized pipe and fittings, 1” IPS and ½” IPS U.S. made chrome plated brass stay open ball valves, foot treadle, and polished stainless steel pull rod.  Unit shall have (2) polypropylene ‘FS Plus’ spray heads with integral “flip-top” dust covers, filters, and 3.2-GPM flow control orifices mounted on a chrome plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall meet or exceed ANSI Z358.1 – 2004, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment G1950HFC and G1950HFC-SSH