- ESBF-7*– Barrier-Free, horizontally mounted emergency shower with ABS plastic shower head (optional stainless steel shower head ”SSH”), 1” IPS Schedule 40 hot dipped galvanized pipe and fittings, 59 ½” long polished stainless steel pull rod, 1” IPS chrome plated brass stay-open ball valve, and stainless steel actuating arm.  Unit shall include ANSI compliant sign.

- Performance: Unit complies with ADA requirements for accessibility by handicapped persons. Unit shall be hydrostatically tested to meet or exceed ANSI Z358.1 – 2004, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment GBF1643 and GBF1643SSH