- EFBF-15* – Barrier Free, recessed wall mounted, swing down eye/face wash with stainless steel housing, ½” IPS brass rotating plug-type valve with Teflon coated O-ring seals. Supplied with in-line strainer to prevent debris from affecting the valve. Unit shall have (2) polypropylene ‘FS Plus’ spray heads with integral filters and 3.2-GPM flow control orifices mounted on a chrome plated brass “panic bar” assembly.  Activate valve by rotating 90° from stored position.  Unit shall include ANSI compliant sign.

- Performance: Unit complies with ADA requirements for accessibility by handicapped persons.  Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2004, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment GBF1735