- TMV -2 – Thermostatic mixing valve (factory set to 85 degrees) for single emergency eye wash.  Unit shall include a built-in cold water by-pass, rough bronze finish, solid bimetal thermostat, locking temperature regulator with limit stop factory set for 90 degrees, integral check stops, dial thermometer, and recessed stainless steel cabinet with door and lock (optional surface mounted cabinet G3602).

- Performance:  Unit shall have a flow range of 0.5-GPM to 5-GPM with a maximum pressure loss of 20-PSI and come with a full 1-year warranty.

- Fixture:  Guardian G3607 and G3602