- DHVR-10* - Deck mounted, hand-held drench hose/eye wash with ½” IPS U.S. made chrome plated brass squeeze valve with renewable stainless steel seat and locking clip, insulator handle, stainless steel handle with plastic cover, nylon deck flange with locator guide, and 8ft. PVC hose.  Unit shall have (2) chrome-plated brass ‘GS Plus’ spray heads with integral stainless steel “flip-top” dust covers, filters and 1.8-GPM flow control orifices mounted on a chrome plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Vandal Resistance: All connections shall be secured with setscrews or cement to prevent disassembly.  Unit shall include ANSI compliant sign.

- Performance:  Unit shall be fully factory assembled and hydrostatically tested before delivery to meet or exceed ANSI Z358.1 – 2004, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment GVR5022